http://rredc.nrel.gov/solar/pubs/tmy2/overview.html

User's Manual for TMY2s

Contents

Title Page

Notice

Preface

Acknowledgments

List of Figures

List of Tables

Section 1... Overview

Typical Meteorological Year - A Description

NSRDB - Source of Data for the TMY2s

Methodology

TMY2 Station Classification

Data Elements

Where to Order

References

Section 2... Stations

Locations

NSRDB Classification

TMY2 Classification

Section 3... Data and Format

File Convention

File Header

Hourly Records

Missing Data

Source and Uncertainty Flags

Section 4... Comparison with Long-Term Data Sets

Solar Radiation Comparisons

Heating and Cooling Degree Day Comparisons

References

Appendix A... Procedures for Developing TMY2s

Sandia Method

Weighting and Indice Modifications

El Chichon Years

Leap Years

Preference for Months with Measured Solar Radiation Data

Month Interface Smoothing

Allowance for Missing Data

Data-Filling Methods

Quality Control

Calculation of Illuminance Data

Assignment of Source and Uncertainty Flags

References

References

Appendix B    Key to Present Weather Elements

Present Weather Elements in the TMY2 Format

Appendix C    Unit Conversion Factors

TMY2 Data Files in Compressed Format

http://rredc.nrel.gov/solar/old_data/nsrdb/tmy2/

1961- 1990: Typical Meteorological Year 2

Brief Summary of the TMY2s This directory contains the data files for the typical meteorological year (TMY) data sets derived from the 1961-1990 National Solar Radiation Data Base (NSRDB). Because they are based on more recent and accurate data and will make possible more accurate performance and economic analyses of energy systems, these data sets are recommended for use in place of earlier TMY data sets derived from the 1952-1975 SOLMET/ERSATZ data base. To distinguish between the old and new TMY data sets, the new TMY data sets are referred to as TMY2s. TMY and TMY2 data sets cannot be used interchangeable because of differences in time (solar versus local), formats, elements, and units. Unless they are revised, computer programs designed for TMY data will not work with TMY2 data. The TMY2s are data sets of hourly values of solar radiation and meteorological elements for a 1-year period. Their intended use is for computer simulations of solar energy conversion systems and building systems to facilitate performance comparisons of different system types, configurations, and locations in the United States and its territories. Because they represent typical rather than extreme conditions, they are not suited for designing systems to meet the worst-case conditions occurring at a location. The TMY2 data sets and manual were produced by the National Renewable Energy Laboratory's (NREL's) Analytic Studies Division under the Resource Assessment Program, which is funded and monitored by the U.S. Department of Energy's Office of Solar Energy Conversion. The Data Individual Station Files In Compressed Format: In alphabetical order by state and city. - ~235 KB In numerical order by WBAN number. - ~235 KB All Stations in a Single Compressed File: Mac Format - 32.7 MB Windows Format - 52.8 MB Unix Format - 67.2 MB

APPENDIX A

Procedures for Developing TMY2s

http://rredc.nrel.gov/solar/pubs/tmy2/appendixa.html

Typical Meteorological Year - A Description

A TMY is a data set of hourly values of solar radiation and meteorological elements for a 1-year period. It consists of months selected from individual years and concatenated to form a complete year. The intended use is for computer simulations of solar energy conversion systems and building systems. Because of the selection criteria, TMYs are not appropriate for simulations of wind energy conversion systems.

A TMY provides a standard for hourly data for solar radiation and other meteorological elements that permit performance comparisons of system types and configurations for one or more locations. A TMY in not necessarily a good indicator of conditions over the next year, or even the next 5 years. Rather, it represents conditions judged to be typical over a long period of time, such as 30 years. Because they represent typical rather than extreme conditions, they are not suited for designing systems and their components to meet the worst-case conditions occurring at a location.

Table 1-1. TMY2 Data Elements and Their Degree of Completeness

| Element | Data Completeness |  |
| --- | --- | --- |
|  | Class A | Class B |
| Extraterrestrial Horizontal Radiation | 1 | 1 |
| Extraterrestrial Direct Normal Radiation | 1 | 1 |
| Global Horizontal Radiation | 1 | 1 |
| Direct Normal Radiation | 1 | 1 |
| Diffuse Horizontal Radiation | 1 | 1 |
| Global Horizontal Illuminance | 1 | 1 |
| Direct Normal Illuminance | 1 | 1 |
| Diffuse Horizontal Illuminance | 1 | 1 |
| Zenith Luminance | 1 | 1 |
| Total Sky Cover | 1 | 1 |
| Opaque Sky Cover | 1 | 1 |
| Dry Bulb Temperature | 1 | 1 |
| Dew Point Temperature | 1 | 1 |
| Relative Humidity | 1 | 1 |
| Atmospheric Pressure | 1 | 1 |
| Wind Direction | 1 | 1 |
| Wind Speed | 1 | 1 |
| Horizontal Visibility | 2 | 2, 3, 4 |
| Ceiling Height | 2 | 2, 3, 4 |
| Present Weather | 2 | 2, 3, 4 |
| Precipitable Water | 1 | 1 |
| Broadband Aerosol Optical Depth | 1 | 1 |
| Snow Depth | 1 | 5 |
| Days Since Last Snowfall | 1 | 5 |
| Notes: 1. Serially complete, no missing data. 2. Data may be present only every third hour. 3. Nighttime data may be missing. 4. Data may be missing for up to 47 hours. 5. Serially complete, except for Colorado Springs, CO. |  |  |

http://rredc.nrel.gov/solar/pubs/tmy2/tab2-1a.html

Table 2-1. Station Locations and Classifications, Part A: Alabama through Iowa

| State | City | WBAN No. | Latitude |  | Longitude |  | Elev (m) | Classification |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Deg | Min | Deg | Min |  | NSRDB | TMY2 |
| Alabama |  |  |  |  |  |  |  |  |  |
|  | Birmingham | 13876 | N33 | 34 | W 86 | 45 | 192 | S | A |
|  | Huntsville | 03856 | N34 | 39 | W 86 | 46 | 190 | S | A |
|  | Mobile | 13894 | N30 | 41 | W 88 | 15 | 67 | S | A |
|  | Montgomery | 13895 | N32 | 18 | W 86 | 24 | 62 | P | A |
| Alaska |  |  |  |  |  |  |  |  |  |
|  | Anchorage | 26451 | N61 | 10 | W150 | 1 | 35 | S | A |
|  | Annette | 25308 | N55 | 2 | W131 | 34 | 34 | S | A |
|  | Barrow | 27502 | N71 | 18 | W156 | 47 | 4 | S | A |
|  | Bethel | 26615 | N60 | 47 | W161 | 48 | 46 | S | A |
|  | Bettles | 26533 | N66 | 55 | W151 | 31 | 205 | S | B |
|  | Big Delta | 26415 | N64 | 0 | W145 | 44 | 388 | S | B |
|  | Cold Bay | 25624 | N55 | 12 | W162 | 43 | 29 | S | A |
|  | Fairbanks | 26411 | N64 | 49 | W147 | 52 | 138 | P | A |
|  | Gulkana | 26425 | N62 | 9 | W145 | 27 | 481 | S | B |
|  | King Salmon | 25503 | N58 | 41 | W156 | 39 | 15 | S | A |
|  | Kodiak | 25501 | N57 | 45 | W152 | 20 | 34 | S | A |
|  | Kotzebue | 26616 | N66 | 52 | W162 | 38 | 5 | S | A |
|  | McGrath | 26510 | N62 | 58 | W155 | 37 | 103 | S | A |
|  | Nome | 26617 | N64 | 30 | W165 | 26 | 7 | S | A |
|  | St. Paul Island | 25713 | N57 | 9 | W170 | 13 | 7 | S | A |
|  | Talkeetna | 26528 | N62 | 18 | W150 | 6 | 105 | S | B |
|  | Yakutat | 25339 | N59 | 31 | W139 | 40 | 9 | S | A |
| Arizona |  |  |  |  |  |  |  |  |  |
|  | Flagstaff | 03103 | N35 | 8 | W111 | 40 | 2135 | S | B |
|  | Phoenix | 23183 | N33 | 26 | W112 | 1 | 339 | P | A |
|  | Prescott | 23184 | N34 | 39 | W112 | 26 | 1531 | S | A |
|  | Tucson | 23160 | N32 | 7 | W110 | 56 | 779 | P | A |
| Arkansas |  |  |  |  |  |  |  |  |  |
|  | Fort Smith | 13964 | N35 | 20 | W 94 | 22 | 141 | S | A |
|  | Little Rock | 13963 | N34 | 44 | W 92 | 14 | 81 | S | A |
| California |  |  |  |  |  |  |  |  |  |
|  | Arcata | 24283 | N40 | 59 | W124 | 6 | 69 | S | A |
|  | Bakersfield | 23155 | N35 | 25 | W119 | 3 | 150 | S | A |
|  | Daggett | 23161 | N34 | 52 | W116 | 47 | 588 | P | A |
|  | Fresno | 93193 | N36 | 46 | W119 | 43 | 100 | P | A |
|  | Long Beach | 23129 | N33 | 49 | W118 | 9 | 17 | S | A |
|  | Los Angeles | 23174 | N33 | 56 | W118 | 24 | 32 | P | A |
|  | Sacramento | 23232 | N38 | 31 | W121 | 30 | 8 | S | A |
|  | San Diego | 23188 | N32 | 44 | W117 | 10 | 9 | P | A |
|  | San Francisco | 23234 | N37 | 37 | W122 | 23 | 5 | S | A |
|  | Santa Maria | 23273 | N34 | 54 | W120 | 27 | 72 | P | B |
| Colorado |  |  |  |  |  |  |  |  |  |
|  | Alamosa | 23061 | N37 | 27 | W105 | 52 | 2297 | P | B |
|  | Boulder | 94018 | N40 | 1 | W105 | 15 | 1634 | P | A |
|  | Colorado Springs | 93037 | N38 | 49 | W104 | 43 | 1881 | S | B |
|  | Eagle | 23063 | N39 | 39 | W106 | 55 | 1985 | S | A |
|  | Grand Junction | 23066 | N39 | 7 | W108 | 32 | 1475 | P | A |
|  | Pueblo | 93058 | N38 | 17 | W104 | 31 | 1439 | S | A |
| Connecticut |  |  |  |  |  |  |  |  |  |
|  | Bridgeport | 94702 | N41 | 10 | W 73 | 8 | 2 | S | A |
|  | Hartford | 14740 | N41 | 56 | W 72 | 41 | 55 | S | A |
| Delaware |  |  |  |  |  |  |  |  |  |
|  | Wilmington | 13781 | N39 | 40 | W 75 | 36 | 24 | S | A |
| Florida |  |  |  |  |  |  |  |  |  |
|  | Daytona Beach | 12834 | N29 | 11 | W 81 | 3 | 12 | P | A |
|  | Jacksonville | 13889 | N30 | 30 | W 81 | 42 | 9 | S | A |
|  | Key West | 12836 | N24 | 33 | W 81 | 45 | 1 | S | A |
|  | Miami | 12839 | N25 | 48 | W 80 | 16 | 2 | P | A |
|  | Tallahassee | 93805 | N30 | 23 | W 84 | 22 | 21 | P | A |
|  | Tampa | 12842 | N27 | 58 | W 82 | 32 | 3 | S | A |
|  | West Palm Beach | 12844 | N26 | 41 | W 80 | 6 | 6 | S | A |
| Georgia |  |  |  |  |  |  |  |  |  |
|  | Athens | 13873 | N33 | 57 | W 83 | 19 | 244 | S | A |
|  | Atlanta | 13874 | N33 | 39 | W 84 | 26 | 315 | P | A |
|  | Augusta | 03820 | N33 | 22 | W 81 | 58 | 45 | S | A |
|  | Columbus | 93842 | N32 | 31 | W 84 | 57 | 136 | S | B |
|  | Macon | 03813 | N32 | 42 | W 83 | 39 | 110 | S | A |
|  | Savannah | 03822 | N32 | 8 | W 81 | 12 | 16 | P | A |
| Hawaii |  |  |  |  |  |  |  |  |  |
|  | Hilo | 21504 | N19 | 43 | W155 | 4 | 11 | S | A |
|  | Honolulu | 22521 | N21 | 20 | W157 | 55 | 5 | P | A |
|  | Kahului | 22516 | N20 | 54 | W156 | 26 | 15 | S | B |
|  | Lihue | 22536 | N21 | 59 | W159 | 21 | 45 | S | A |
| Idaho |  |  |  |  |  |  |  |  |  |
|  | Boise | 24131 | N43 | 34 | W116 | 13 | 874 | P | A |
|  | Pocatello | 24156 | N42 | 55 | W112 | 36 | 1365 | S | A |
| Illinois |  |  |  |  |  |  |  |  |  |
|  | Chicago | 94846 | N41 | 47 | W 87 | 45 | 190 | S | A |
|  | Moline | 14923 | N41 | 27 | W 90 | 31 | 181 | S | A |
|  | Peoria | 14842 | N40 | 40 | W 89 | 41 | 199 | S | A |
|  | Rockford | 94822 | N42 | 12 | W 89 | 6 | 221 | S | A |
|  | Springfield | 93822 | N39 | 50 | W 89 | 40 | 187 | S | A |
| Indiana |  |  |  |  |  |  |  |  |  |
|  | Evansville | 93817 | N38 | 3 | W 87 | 32 | 118 | S | A |
|  | Fort Wayne | 14827 | N41 | 0 | W 85 | 12 | 252 | S | A |
|  | Indianapolis | 93819 | N39 | 44 | W 86 | 17 | 246 | P | A |
|  | South Bend | 14848 | N41 | 42 | W 86 | 19 | 236 | S | A |
| Iowa |  |  |  |  |  |  |  |  |  |
|  | Des Moines | 14933 | N41 | 32 | W 93 | 39 | 294 | S | A |
|  | Mason City | 14940 | N43 | 9 | W 93 | 20 | 373 | S | A |
|  | Sioux City | 14943 | N42 | 24 | W 96 | 23 | 336 | S | A |
|  | Waterloo | 94910 | N42 | 33 | W 92 | 24 | 265 | S | A |

Table 3-1. Header Elements in the TMY2 Format
(For First Record of Each File)

| Field Position | Element | Definition |
| --- | --- | --- |
| 002 - 006 | WBAN Number | Station's Weather Bureau Army Navy number (see Table 2-1) |
| 008 - 029 | City | City where the station is located (maximum of 22 characters) |
| 031 - 032 | State | State where the station is located (abbreviated to two letters) |
| 034 - 036 | Time Zone | Time zone is the number of hours by which the local standard time is ahead of or behind Universal Time. For example, Mountain Standard Time is designated -7 because it is 7 hours behind Universal Time. |
| 038 - 044 038 040 - 041 043 - 044 | Latitude | Latitude of the station N = North of equator Degrees Minutes |
| 046 - 053 046 048 - 050 052 - 053 | Longitude | Longitude of the station W = West, E = East Degrees Minutes |
| 056 - 059 | Elevation | Elevation of station in meters above sea level |
| FORTRAN Sample Format: ( 1X,A5,1X,A22,1X,A2,1X I3,1X,Al,lX,I2,1X,I2,1X,Al,lX,I3,1X,I2,2X,I4 ) C Sample Format: ( %s %s %s %d %s %d %d %s %d %d %d ) |  |  |

http://rredc.nrel.gov/solar/pubs/tmy2/tab3-2.html

Table 3-2. Data Elements in the TMY2 Format
(For All Except the First Record)

| Field Position | Element | Values | Definition |
| --- | --- | --- | --- |
| 002 - 009 002 - 003 004 - 005 006 - 007 008 - 009 | Local Standard Time Year Month Day Hour | 61 - 90 1 - 12 1 - 31 1 - 24 | Year, 1961-1990 Month Day of month Hour of day in local standard time |
| 010 - 013 | Extraterrestrial Horizontal Radiation | 0 - 1415 | Amount of solar radiation in Wh/m2 received on a horizontal surface at the top of the atmosphere during the 60 minutes preceding the hour indicated |
| 014 - 017 | Extraterrestrial Direct Normal Radiation | 0 - 1415 | Amount of solar radiation in Wh/m2 received on a surface normal to the sun at the top of the atmosphere during the 60 minutes preceding the hour indicated |
| 018 - 023 018 - 021 022 023 | Global Horizontal Radiation Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 1200 A - H, ? 0 - 9 | Total amount of direct and diffuse solar radiation in Wh/m2 received on a horizontal surface during the 60 minutes preceding the hour indicated |
| 024 - 029 024 - 027 028 029 | Direct Normal Radiation Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 1100 A - H, ? 0 - 9 | Amount of solar radiation in Wh/m2 received within a 5.7° field of view centered on the sun during the 60 minutes preceding the hour indicated |
| 030 - 035 030 - 033 034 035 | Diffuse Horizontal Radiation Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 700 A - H, ? 0 - 9 | Amount of solar radiation in Wh/m2 received from the sky (excluding the solar disk) on a horizontal surface during the 60 minutes preceding the hour indicated |
| 036 - 041 036 - 039 040 041 | Global Horiz. Illuminance Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 1300 I, ? 0 - 9 | Average total amount of direct and diffuse illuminance in hundreds of lux received on a horizontal surface during the 60 minutes preceding the hour indicated 0 to 1300 = 0 to 130,000 lux |
| 042 - 047 042 - 045 046 047 | Direct Normal Illuminance Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 1100 I, ? 0 - 9 | Average amount of direct normal illuminance in hundreds of lux received within a 5.7° field of view centered on the sun during the 60 minutes preceding the hour indicated. 0 to 1100 = 0 to 110,000 lux |
| 048 - 053 048 - 051 052 053 | Diffuse Horiz. Illuminance Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 800 I, ? 0 - 9 | Average amount of illuminance in hundreds of lux received from the sky (excluding the solar disk) on a horizontal surface during the 60 minutes preceding the hour indicated. 0 to 800= 0 to 80,000 lux |
| 054 - 059 054 - 057 058 059 | Zenith Luminance Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 7000 I, ? 0 - 9 | Average amount of luminance at the sky's zenith in tens of Cd/m2 during the 60 minutes preceding the hour indicated. 0 to 7000 = 0 to 70,000 Cd/m2 |
| 060 - 063 060 - 061 062 063 | Total Sky Cover Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 10 A - F 0 - 9 | Amount of sky dome in tenths covered by clouds or obscuring phenomena at the hour indicated |
| 064 - 067 064 - 065 066 067 | Opaque Sky Cover Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 10 A - F 0 - 9 | Amount of sky dome in tenths covered by clouds or obscuring phenomena that prevent observing the sky or higher cloud layers at the hour indicated |
| 068 - 073 068 - 071 072 073 | Dry Bulb Temperature Data Value Flag for Data Source Flag for Data Uncertainty | -500 to 500 A - F 0 - 9 | Dry bulb temperature in tenths of °C at the hour indicated -500 to 500 = -50.0 to 50.0°C |
| 074 - 079 074 - 077 078 079 | Dew Point Temperature Data Value Flag for Data Source Flag for Data Uncertainty | -600 to 300 A - F 0 - 9 | Dew point temperature in tenths of °C at the hour indicated -600 to 300 = -60.0 to 30.0°C |
| 080 - 084 080 - 082 083 084 | Relative Humidity Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 100 A - F 0 - 9 | Relative humidity in percent at the hour indicated |
| 085 - 090 085 - 088 089 090 | Atmospheric Pressure Data Value Flag for Data Source Flag for Data Uncertainty | 700 - 1100 A - F 0 - 9 | Atmospheric pressure at station in millibars at the hour indicated |
| 091 - 095 091 - 093 094 095 | Wind Direction Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 360 A - F 0 - 9 | Wind direction in degrees at the hour indicated. ( N = 0 or 360, E = 90, S = 180,W = 270 ). For calm winds, wind direction equals zero. |
| 096 - 100 096 - 098 099 100 | Wind Speed Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 400 A - F 0 - 9 | Wind speed in tenths of meters per second at the hour indicated. 0 to 400 = 0 to 40.0 m/s |
| 101 - 106 101 - 104 105 106 | Visibility Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 1609 A - F, ? 0 - 9 | Horizontal visibility in tenths of kilometers at the hour indicated. 7777 = unlimited visibility 0 to 1609 = 0.0 to 160.9 km 9999 = missing data |
| 107 - 113 107 - 111 112 113 | Ceiling Height Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 30450 A - F, ? 0 - 9 | Ceiling height in meters at the hour indicated. 77777 = unlimited ceiling height 88888 = cirroform 99999 = missing data |
| 114 - 123 | Present Weather | See Appendix B | Present weather conditions denoted by a 10-digit number. See Appendix B for key to present weather elements. |
| 124 - 128 124 - 126 127 128 | Precipitable Water Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 100 A - F 0 - 9 | Precipitable water in millimeters at the hour indicated |
| 129 - 133 129 - 131 132 133 | Aerosol Optical Depth Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 240 A - F 0 - 9 | Broadband aerosol optical depth (broad-band turbidity) in thousandths on the day indicated. 0 to 240 = 0.0 to 0.240 |
| 134 - 138 134 - 136 137 138 | Snow Depth Data Value Flag for Data Source Flag for Data Uncertainty | 0- 150 A - F, ? 0 - 9 | Snow depth in centimeters on the day indicated. 999 = missing data |
| 139 - 142 139 - 140 141 142 | Days Since Last Snowfall Data Value Flag for Data Source Flag for Data Uncertainty | 0 - 88 A - F, ? 0 - 9 | Number of days since last snowfall. 88 = 88 or greater days 99 = missing data |
| FORTRAN Sample Format: (1X, 4I2, 2I4, 7 (I4, A1, I1), 2( I2, A1, I1), 2 (I4, A1, I1), 1(I3, A1, I1), 1( I4, A1, I1), 2 (I3, A1, I1), 1(I4, A1, I1), 1(I5, A1, I1), 10I1, 3 (I3, A1, I1), 1(I2, A1, I1)) C Sample Format: (%2d%2d%2d%2d%4d%4d%4d%1s%1d%4d%1s%1d%4d%1s%1d%4d%1s%1d%4d%1s%1d%4d%1s %1d%4d%1s%1d%2d%1s%1d%2d%1s%1d%4d%1s%1d%4d%1s%1d%3d%1s%1d%4d%1s%1d%3d %1s%1d%3d%1s%1d%4d%1s%1d%5ld%1s%1d%1d%1d%1d%1d%1d%1d%1d%1d%1d%1d%3d%1s %1d%3d%1s%1d%3d%1s%1d%2d%1s%1d) Note: For ceiling height data, integer variable should accept data values as large as 99999. |  |  |  |

http://rredc.nrel.gov/solar/pubs/tmy2/data.html#missing

Table 3-3. Solar Radiation and Illuminance Source Flags

| Flag | Definition |
| --- | --- |
| A | Post-1976 measured solar radiation data as received from NCDC or other sources |
| B | Same as "A" except the global horizontal data underwent a calibration correction |
| C | Pre-1976 measured global horizontal data (direct and diffuse were not measured before 1976), adjusted from solar to local time, usually with a calibration correction |
| D | Data derived from the other two elements of solar radiation using the relationship, global = diffuse + direct x cosine(zenith) |
| E | Modeled solar radiation data using inputs of observed sky cover (cloud amount) and aerosol optical depths derived from direct normal data collected at the same location |
| F | Modeled solar radiation data using interpolated sky cover and aerosol optical depths derived from direct normal data collected at the same location |
| G | Modeled solar radiation data using observed sky cover and aerosol optical depths estimated from geographical relationships |
| H | Modeled solar radiation data using interpolated sky cover and estimated aerosol optical depths |
| I | Modeled illuminance or luminance data derived from measured or modeled solar radiation data |
| ? | Source does not fit any of the above categories. Used for nighttime values and missing data |

Table 3-4. Solar Radiation and Illuminance Uncertainty Flags

| Flag | Uncertainty Range (%) |
| --- | --- |
| 1 | Not used |
| 2 | 2 - 4 |
| 3 | 4 - 6 |
| 4 | 6 - 9 |
| 5 | 9 - 13 |
| 6 | 13 - 18 |
| 7 | 18 - 25 |
| 8 | 25 - 35 |
| 9 | 35 - 50 |
| 0 | Not applicable |

Table 3-5. Meteorological Source Flags

| Flag | Definition |
| --- | --- |
| A | Data as received from NCDC, converted to SI units |
| B | Linearly interpolated |
| C | Non-linearly interpolated to fill data gaps from 6 to 47 hours in length |
| D | Not used |
| E | Modeled or estimated, except: precipitable water, calculated from radiosonde data; dew point temperature calculated from dry bulb temperature and relative humidity; and relative humidity calculated from dry bulb temperature and dew point temperature |
| F | Precipitable water, calculated from surface vapor pressure; aerosol optical depth, estimated from geographic correlation |
| ? | Source does not fit any of the above. Used mostly for missing data |

Table 3-6.Meteorological Uncertainty Flags

| Flag | Definition |
| --- | --- |
| 1 - 6 | Not used |
| 7 | Uncertainty consistent with NWS practices and the instrument or observation used to obtain the data |
| 8 | Greater uncertainty than 7 because values were interpolated or estimated |
| 9 | Greater uncertainty than 8 or unknown |
| 0 | Not definable |

http://rredc.nrel.gov/solar/pubs/tmy2/fig3-1.html

14944 SIOUX_FALLS            SD  -6 N 43 34 W  96 44   435

85010101000000000000?00000?00000?00000?00000?00000?00000?010A710A7-150A7-211A7060A70975A7360A7052A70161A700945A70999099999004E7050F8000A700E7

85010102000000000000?00000?00000?00000?00000?00000?00000?0l0A710A7-144A7-206A7060A70975A7350A7077A70161A700914A70999099999004E7050F8000A700E7

85010103000000000000?00000?00000?00000?00000?00000?00000?0l0A710A7-144A7-200A7063A70975A7340A7062A70161A700732A70999099999004E7050F8000A700E7

85010104000000000000?00000?00000?00000?00000?00000?00000?0l0A710A7-150A7-206A7063A70976A7330A7072A70161A700640A70999099999004E7050F8000A700E7

85010105000000000000?00000?00000?00000?00000?00000?00000?010A710A7-156A7-217A7060A70976A7330A7067A70161A700640A70999099999003E7050F8000A700E7

85010106000000000000?00000?00000?00000?00000?00000?00000?0l0A710A7-167A7-222A7062A70976A7340A7067A70161A700640A70999099999003E7050F8000A700E7

85010107000000000000?00000?00000?00000?00000?00000?00000?004A704A7-183A7-233A7065A70977A7300A7052A70193A777777A70999999999003E7050F8000A700E7

85010108000000000000?00000?00000?00000?00000?00000?00000?002A702A7-194A7-244A7065A70978A7310A7036A70193A777777A70999999999003E7050F8000A700E7

85010109010212970037G50173G40024G50038I5007lI40033I50043I604A700A7-200A7-256A7062A70978A7330A7046A70193A777777A70999999999003E7050F8000A700E7

85010110028714150157G50560G40043G50159I50444I40069I50079I600A700A7-189A7-256A7056A70979A7310A7067A70193A777777A70999999999003E7050F8000A700E7

85010111043614150276G40714G40056G50286I40642I40088I50111I500A700A7-172A7-250A7051A70979A7310A7062A7016lA777777A70999999999003E7050F8000A700E7

85010112053014150357G40782G40064G50374I40735I40098I5013lI500A700A7-167A7-244A7051A70978A7300A7062A70161A777777A70999999999003E7050F8000A700E7

85010113056214150387G40806G40067G50407I40767I40101I50139I500A700A7-156A7-244A7047A70978A7320A7067A70193A777777A70999999999003E7050F8000A700E7

85010114053014150359G40788G40064G50377I40742I40098I5013lI500A700A7-144A7-239A7045A70978A7310A7062A70193A777777A70999999999003E7050F8000A700E7

85010115043614150277G40716G40056G50289I40645I40088I50111I500A700A7-139A7-239A7043A70978A7330A7052A70193A777777A70999999999003E7050F8000A700E7

85010116028614150157G50564G40043G50162I50450I40069I50080I600A700A7-139A7-233A7045A70978A7300A7052A70161A777777A70999999999003E7050F8000A700E7

85010117010412730038G50209G4002lG50038I50104I40030I50038I600A700A7-150A7-233A7049A70978A7290A7041A70241A777777A70999999999003E7050F8000A700E7

85010118000000000000?00000?00000?00000?00000?00000?00000?000A700A7-167A7-233A7057A70978A7000A7000A70241A777777A70999999999003E7050F8000A700E7

85010119000000000000?00000?00000?00000?00000?00000?00000?000A700A7-172A7-233A7059A70978A7000A7000A70241A777777A70999999999003E7050F8000A700E7

85010120000000000000?00000?00000?00000?00000?00000?00000?000A700A7-178A7-233A7062A70978A7000A7000A70241A777777A70999999999003E7050F8000A700E7

85010121000000000000?00000?00000?00000?00000?00000?00000?000A700A7-183A7-239A7062A70978A7260A7015A70241A777777A70999999999003E7050F8000A700E7

85010122000000000000?00000?00000?00000?00000?00000?00000?000A700A7-183A7-239A7062A70977A7220A702lA70241A777777A70999999999003E7050F8000A700E7

85010123000000000000?00000?00000?00000?00000?00000?00000?000A700A7-178A7-239A7059A70977A7220A7015A70241A777777A70999999999003E7050F8000A700E7

85010124000000000000?00000?00000?00000?00000?00000?00000?000A700A7-178A7-239A7059A70977A7240A7010A70241A777777A70999999999003E7050F8000A700E7

1         1         1         1         1

1         2         3         4         5         6         7         8         9         0         1         2         3         4

1234567890123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890123456789012345678901234567890123456789012

(for field position identification only)

Figure 3-1. Sample file header and data in the TMY2 format for January 1